Топология по лабе:

Работа со switch2 – настройка его ip

Switch2(config)#interface vlan 72 Switch2(config-if)#ip address 192.168.13.10 255.255.255.0 Switch2(config-if)#no shutdown Switch2(config-if)#do show ip interface brief Interface IP-Address OK? Method Status Protocol FastEthernet0/1 unassigned YES manual up up FastEthernet0/2 unassigned YES manual up up FastEthernet0/3 unassigned YES manual up up FastEthernet0/4 unassigned YES manual down down FastEthernet0/5 unassigned YES manual down down FastEthernet0/6 unassigned YES manual down down FastEthernet0/7 unassigned YES manual down down FastEthernet0/8 unassigned YES manual down down FastEthernet0/9 unassigned YES manual down down FastEthernet0/10 unassigned YES manual down down FastEthernet0/11 unassigned YES manual down down FastEthernet0/12 unassigned YES manual down down FastEthernet0/13 unassigned YES manual down down FastEthernet0/14 unassigned YES manual down down FastEthernet0/15 unassigned YES manual down down FastEthernet0/16 unassigned YES manual down down FastEthernet0/17 unassigned YES manual down down FastEthernet0/18 unassigned YES manual down down FastEthernet0/19 unassigned YES manual down down FastEthernet0/20 unassigned YES manual down down FastEthernet0/21 unassigned YES manual down down FastEthernet0/22 unassigned YES manual down down FastEthernet0/23 unassigned YES manual down down FastEthernet0/24 unassigned YES manual down down GigabitEthernet0/1 unassigned YES manual down down GigabitEthernet0/2 unassigned YES manual down down Vlan1 unassigned YES manual administratively down down Vlan72 192.168.13.10 YES manual down down VLAN интерфейс показывает статус UP только когда есть активные порты в этом VLAN Switch2(config)#interface fastethernet 0/1 Switch2(config-if)#switchport mode access Switch2(config-if)#switchport access vlan 72 % Access VLAN does not exist. Creating vlan 72 Switch2(config-if)# %LINK-5-CHANGED: Interface Vlan72, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan72, changed state to up Switch2(config-if)#no shutdown Switch2(config)#interface fastethernet 0/3 Switch2(config-if)#switchport mode access Switch2(config-if)#switchport access vlan 72 Switch2(config-if)#no shutdown Switch2(config-if)#do show ip interface brief Interface IP-Address OK? Method Status Protocol FastEthernet0/1 unassigned YES manual up up FastEthernet0/2 unassigned YES manual up up FastEthernet0/3 unassigned YES manual up up FastEthernet0/4 unassigned YES manual down down FastEthernet0/5 unassigned YES manual down down FastEthernet0/6 unassigned YES manual down down FastEthernet0/7 unassigned YES manual down down FastEthernet0/8 unassigned YES manual down down FastEthernet0/9 unassigned YES manual down down FastEthernet0/10 unassigned YES manual down down FastEthernet0/11 unassigned YES manual down down FastEthernet0/12 unassigned YES manual down down FastEthernet0/13 unassigned YES manual down down FastEthernet0/14 unassigned YES manual down down FastEthernet0/15 unassigned YES manual down down FastEthernet0/16 unassigned YES manual down down FastEthernet0/17 unassigned YES manual down down FastEthernet0/18 unassigned YES manual down down FastEthernet0/19 unassigned YES manual down down FastEthernet0/20 unassigned YES manual down down FastEthernet0/21 unassigned YES manual down down FastEthernet0/22 unassigned YES manual down down FastEthernet0/23 unassigned YES manual down down FastEthernet0/24 unassigned YES manual down down GigabitEthernet0/1 unassigned YES manual down down GigabitEthernet0/2 unassigned YES manual down down Vlan1 unassigned YES manual administratively down down Vlan72 192.168.13.10 YES manual up up

Настраиваю ip для pc0 pc1

switch 192.168.13.10

pc0 - 192.168.13.20

pc1 - 192.168.13.21

Настройка switch1

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch1 Switch1(config)#interface vlan 27 Switch1(config-if)#ip address 192.168.14.11 255.255.255.0 Switch1(config-if)#do show ip interface brief Interface IP-Address OK? Method Status Protocol FastEthernet0/1 unassigned YES manual up up FastEthernet0/2 unassigned YES manual up up FastEthernet0/3 unassigned YES manual down down FastEthernet0/4 unassigned YES manual down down FastEthernet0/5 unassigned YES manual down down FastEthernet0/6 unassigned YES manual down down FastEthernet0/7 unassigned YES manual down down FastEthernet0/8 unassigned YES manual down down FastEthernet0/9 unassigned YES manual down down FastEthernet0/10 unassigned YES manual down down FastEthernet0/11 unassigned YES manual down down FastEthernet0/12 unassigned YES manual down down FastEthernet0/13 unassigned YES manual down down FastEthernet0/14 unassigned YES manual down down FastEthernet0/15 unassigned YES manual down down FastEthernet0/16 unassigned YES manual down down FastEthernet0/17 unassigned YES manual down down FastEthernet0/18 unassigned YES manual down down FastEthernet0/19 unassigned YES manual down down FastEthernet0/20 unassigned YES manual down down FastEthernet0/21 unassigned YES manual down down FastEthernet0/22 unassigned YES manual down down FastEthernet0/23 unassigned YES manual down down FastEthernet0/24 unassigned YES manual down down GigabitEthernet0/1 unassigned YES manual down down GigabitEthernet0/2 unassigned YES manual down down Vlan1 unassigned YES manual administratively down down Vlan27 192.168.14.11 YES manual down down Switch1(config-if)#exit Switch1(config)#interface fastethernet 0/2 Switch1(config-if)#switchport mode access Switch1(config-if)#switchport access vlan 27 % Access VLAN does not exist. Creating vlan 27 Switch1(config-if)# %LINK-5-CHANGED: Interface Vlan27, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan27, changed state to up Switch1(config-if)#no shutdown Switch1(config-if)#do show ip interface brief Interface IP-Address OK? Method Status Protocol FastEthernet0/1 unassigned YES manual up up FastEthernet0/2 unassigned YES manual up up FastEthernet0/3 unassigned YES manual down down FastEthernet0/4 unassigned YES manual down down FastEthernet0/5 unassigned YES manual down down FastEthernet0/6 unassigned YES manual down down FastEthernet0/7 unassigned YES manual down down FastEthernet0/8 unassigned YES manual down down FastEthernet0/9 unassigned YES manual down down FastEthernet0/10 unassigned YES manual down down FastEthernet0/11 unassigned YES manual down down FastEthernet0/12 unassigned YES manual down down FastEthernet0/13 unassigned YES manual down down FastEthernet0/14 unassigned YES manual down down FastEthernet0/15 unassigned YES manual down down FastEthernet0/16 unassigned YES manual down down FastEthernet0/17 unassigned YES manual down down FastEthernet0/18 unassigned YES manual down down FastEthernet0/19 unassigned YES manual down down FastEthernet0/20 unassigned YES manual down down FastEthernet0/21 unassigned YES manual down down FastEthernet0/22 unassigned YES manual down down FastEthernet0/23 unassigned YES manual down down FastEthernet0/24 unassigned YES manual down down GigabitEthernet0/1 unassigned YES manual down down GigabitEthernet0/2 unassigned YES manual down down Vlan1 unassigned YES manual administratively down down Vlan27 192.168.13.11 YES manual up up

SWITCH 0

Switch0>enable Switch0#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch0(config)#interface vlan 72 Switch0(config-if)#ip address 192.168.13.12 255.255.255.0 Switch0(config-if)#exit Switch0(config)#interface fastethernet 0/2 Switch0(config-if)#switchport mode trunk Switch0(config-if)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up Switch0(config-if)#switchport trunk allowed vlan 72,27 Switch0(config-if)#no shutdown Switch0(config)#ip default-gateway 192.168.13.1 Switch0(config)#interface vlan 72 Switch0(config-if)#no shutdown Switch0(config-if)#exit Создать VLAN на Switch0 Switch0(config)#vlan 72 Switch0(config-vlan)# %LINK-5-CHANGED: Interface Vlan72, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan72, changed state to up Switch0(config-vlan)#exit Switch0(config)#vlan 27 Switch0(config-vlan)#exit Switch0(config)#do show interface trunk Port Mode Encapsulation Status Native vlan Fa0/1 on 802.1q trunking 1 Fa0/2 on 802.1q trunking 1 Fa0/3 on 802.1q trunking 1 Port Vlans allowed on trunk Fa0/1 27,72 Fa0/2 27,72 Fa0/3 27,72 Port Vlans allowed and active in management domain Fa0/1 27,72 Fa0/2 27,72 Fa0/3 27,72 Port Vlans in spanning tree forwarding state and not pruned Fa0/1 none Fa0/2 none Fa0/3 none

SWITCH 2

Switch2>enable Switch2#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch2(config)#interface fastethernet 0/2 Switch2(config-if)#switchport mode trunk Switch2(config-if)#switchport trunk allowed vlan 72,27 Switch2(config-if)#no shutdown Switch2(config-if)#do show ip interface brief Interface IP-Address OK? Method Status Protocol FastEthernet0/1 unassigned YES manual up up FastEthernet0/2 unassigned YES manual up up FastEthernet0/3 unassigned YES manual up up FastEthernet0/4 unassigned YES manual down down FastEthernet0/5 unassigned YES manual down down FastEthernet0/6 unassigned YES manual down down FastEthernet0/7 unassigned YES manual down down FastEthernet0/8 unassigned YES manual down down FastEthernet0/9 unassigned YES manual down down FastEthernet0/10 unassigned YES manual down down FastEthernet0/11 unassigned YES manual down down FastEthernet0/12 unassigned YES manual down down FastEthernet0/13 unassigned YES manual down down FastEthernet0/14 unassigned YES manual down down FastEthernet0/15 unassigned YES manual down down FastEthernet0/16 unassigned YES manual down down FastEthernet0/17 unassigned YES manual down down FastEthernet0/18 unassigned YES manual down down FastEthernet0/19 unassigned YES manual down down FastEthernet0/20 unassigned YES manual down down FastEthernet0/21 unassigned YES manual down down FastEthernet0/22 unassigned YES manual down down FastEthernet0/23 unassigned YES manual down down FastEthernet0/24 unassigned YES manual down down GigabitEthernet0/1 unassigned YES manual down down GigabitEthernet0/2 unassigned YES manual down down Vlan1 unassigned YES manual administratively down down Vlan72 192.168.13.10 YES manual up up Switch2(config)#ip default-gateway 192.168.13.1

SWITCH 1

Switch1(config)#interface fastethernet 0/1 Switch1(config-if)#switchport mode trunk Switch1(config-if)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up Switch1(config-if)#switchport trunk allowed vlan 72,27 Switch1(config-if)#no shutdown Switch1(config-if)#do show ip interface brief Interface IP-Address OK? Method Status Protocol FastEthernet0/1 unassigned YES manual up up FastEthernet0/2 unassigned YES manual up up FastEthernet0/3 unassigned YES manual down down FastEthernet0/4 unassigned YES manual down down FastEthernet0/5 unassigned YES manual down down FastEthernet0/6 unassigned YES manual down down FastEthernet0/7 unassigned YES manual down down FastEthernet0/8 unassigned YES manual down down FastEthernet0/9 unassigned YES manual down down FastEthernet0/10 unassigned YES manual down down FastEthernet0/11 unassigned YES manual down down FastEthernet0/12 unassigned YES manual down down FastEthernet0/13 unassigned YES manual down down FastEthernet0/14 unassigned YES manual down down FastEthernet0/15 unassigned YES manual down down FastEthernet0/16 unassigned YES manual down down FastEthernet0/17 unassigned YES manual down down FastEthernet0/18 unassigned YES manual down down FastEthernet0/19 unassigned YES manual down down FastEthernet0/20 unassigned YES manual down down FastEthernet0/21 unassigned YES manual down down FastEthernet0/22 unassigned YES manual down down FastEthernet0/23 unassigned YES manual down down FastEthernet0/24 unassigned YES manual down down GigabitEthernet0/1 unassigned YES manual down down GigabitEthernet0/2 unassigned YES manual down down Vlan1 unassigned YES manual administratively down down Vlan27 192.168.14.11 YES manual up up Switch1(config)#ip default-gateway 192.168.14.1

Настроить trunk к Router0 на Switch0

Switch0(config)#ip default-gateway 192.168.13.1 Switch0(config)#interface fastethernet 0/3 Switch0(config-if)#switchport mode trunk Switch0(config-if)#switchport trunk allowed vlan 72,27 Switch0(config-if)#no shutdown

Настройка Router0

Router> enable Router# configure terminal ! Суб-интерфейс для VLAN 72 Router(config)# interface gigabitethernet 0/0.72 Router(config-subif)# encapsulation dot1Q 72 Router(config-subif)# ip address 192.168.13.1 255.255.255.0 Router(config-subif)# no shutdown Router(config-subif)# exit ! Суб-интерфейс для VLAN 27 Router(config)# interface gigabitethernet 0/0.27 Router(config-subif)# encapsulation dot1Q 27 Router(config-subif)# ip address 192.168.14.1 255.255.255.0 Router(config-subif)# no shutdown Router(config-subif)# exit ! Основной интерфейс Router(config)# interface gigabitethernet 0/0 Router(config-if)# no shutdown Router(config-if)# exit Router#show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0 unassigned YES unset up up GigabitEthernet0/0.27 192.168.14.1 YES manual up up GigabitEthernet0/0.72 192.168.13.1 YES manual up up GigabitEthernet0/1 unassigned YES unset administratively down down GigabitEthernet0/2 unassigned YES unset administratively down down Vlan1 unassigned YES unset administratively down down Router#show running-config interface GigabitEthernet0/0 no ip address duplex auto speed auto ! interface GigabitEthernet0/0.27 encapsulation dot1Q 27 ip address 192.168.14.1 255.255.255.0 ! interface GigabitEthernet0/0.72 encapsulation dot1Q 72 ip address 192.168.13.1 255.255.255.0 ! interface GigabitEthernet0/1 no ip address duplex auto speed auto shutdown ! interface GigabitEthernet0/2 no ip address duplex auto speed auto shutdown ! interface Vlan1 no ip address shutdown ! ip classless ! ip flow-export version 9

Switch4

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch4 Switch4(config)#vlan 72 Switch4(config-vlan)#name VLAN72_Network Switch4(config-vlan)#exit Switch4(config)#interface vlan 72 Switch4(config-if)# %LINK-5-CHANGED: Interface Vlan72, changed state to up Switch4(config-if)#ip address 192.168.15.14 255.255.255.0 Switch4(config-if)#no shutdown Switch4(config-if)#exit Switch4(config)# ip default-gateway 192.168.15.1 Switch4(config)#interface range fastethernet 0/2-3 Switch4(config-if-range)#switchport mode access Switch4(config-if-range)#switchport access vlan 72 Switch4(config-if-range)# %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan72, changed state to up Switch4(config-if-range)#no shutdown Switch4(config-if-range)#exit Switch4(config)#interface fastethernet 0/1 Switch4(config-if)#switchport mode trunk Switch4(config-if)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up Switch4(config-if)#switchport trunk allowed vlan 72,27 Switch4(config-if)#no shutdown Switch4(config-if)#exit

Switch5

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch5 Switch5(config)#vlan 27 Switch5(config-vlan)#name VLAN27_Network Switch5(config-vlan)#exit Switch5(config)#interface fastethernet 0/1 Switch5(config-if)#switchport mode access Switch5(config-if)#switchport access vlan 27 Switch5(config-if)#no shutdown Switch5(config-if)#exit Switch5(config)#interface fastethernet 0/2 Switch5(config-if)#switchport mode trunk Switch5(config-if)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up Switch5(config-if)#switchport trunk allowed vlan 72,27 Switch5(config-if)#no shutdown Switch5(config-if)#exit Switch5(config)#interface vlan 27 Switch5(config-if)# %LINK-5-CHANGED: Interface Vlan27, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan27, changed state to up Switch5(config-if)#ip address 192.168.16.15 255.255.255.0 Switch5(config-if)#no shutdown Switch5(config-if)#exit Switch5(config)# ip default-gateway 192.168.16.1

Switch6

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch6 Switch6(config)#vlan 27 Switch6(config-vlan)#name VLAN27_Network Switch6(config-vlan)#exit Switch6(config)#interface fastethernet 0/1 Switch6(config-if)#switchport mode access Switch6(config-if)#switchport access vlan 27 Switch6(config-if)#no shutdown Switch6(config-if)#exit Switch6(config)#interface fastethernet 0/3 Switch6(config-if)#switchport mode trunk Switch6(config-if)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up Switch6(config-if)#switchport trunk allowed vlan 72,27 Switch6(config-if)#no shutdown Switch6(config-if)#exit Switch6(config)#interface vlan 27 Switch6(config-if)# %LINK-5-CHANGED: Interface Vlan27, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan27, changed state to up Switch6(config-if)#ip address 192.168.16.16 255.255.255.0 Switch6(config-if)#no shutdown Switch6(config-if)#exit Switch6(config)# ip default-gateway 192.168.16.1

Switch3

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch3 Switch3(config)#vlan 72 Switch3(config-vlan)#name VLAN72_Network Switch3(config-vlan)#exit Switch3(config)#vlan 27 Switch3(config-vlan)#name VLAN27_Network Switch3(config-vlan)#exit Switch3(config)#interface gigabitethernet 0/1 Switch3(config-if)#switchport mode trunk Switch3(config-if)#switchport trunk allowed vlan 72,27 Switch3(config-if)#no shutdown Switch3(config-if)#exit Switch3(config)#interface range fastethernet 0/1-3 Switch3(config-if-range)#switchport mode trunk Switch3(config-if-range)#switchport trunk allowed vlan 72,27 Switch3(config-if-range)#no shutdown Switch3(config-if-range)#exit Switch3(config)#interface vlan 72 Switch3(config-if)# %LINK-5-CHANGED: Interface Vlan72, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan72, changed state to up Switch3(config-if)#ip address 192.168.13.13 255.255.255.0 Switch3(config-if)#no shutdown Switch3(config-if)#exit Switch3(config)#ip default-gateway 192.168.15.1

Router

Router(config)#interface gigabitethernet 0/1.72 Router(config-subif)#encapsulation dot1Q 72 Router(config-subif)#ip address 192.168.15.1 255.255.255.0 Router(config-subif)#no shutdown Router(config-subif)#exit Router(config)#interface gigabitethernet 0/1.27 Router(config-subif)#encapsulation dot1Q 72 %Configuration of multiple subinterfaces of the same main interface with the same VID (72) is not permitted. This VID is already configured on GigabitEthernet0/1.72. Router(config-subif)#encapsulation dot1Q 27 Router(config-subif)#ip address 192.168.16.1 255.255.255.0 Router(config-subif)#no shutdown Router(config-subif)#exit Router(config)#do show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0 unassigned YES unset up up GigabitEthernet0/0.27 192.168.14.1 YES manual up up GigabitEthernet0/0.72 192.168.13.1 YES manual up up GigabitEthernet0/1 unassigned YES unset up up GigabitEthernet0/1.27 192.168.16.1 YES manual up up GigabitEthernet0/1.72 192.168.15.1 YES manual up up GigabitEthernet0/2 unassigned YES unset administratively down down Vlan1 unassigned YES unset administratively down down

https://chat.deepseek.com/a/chat/s/30e026de-34b3-48fa-8d5e-8bcd2f2137eb

Аутентификация

Switch2(config)#username meadmin privilege 15 password 1234 Switch2(config)#line vty 0 15 Switch2(config-line)#login local Switch2(config-line)#transport input telnet Switch2(config-line)#exit

copy running-config startup-config

Пример с PC0

C:\>telnet 192.168.13.10 Trying 192.168.13.10 ...Open User Access Verification Username: meadmin 1234

Разница между Access и Trunk портами.

Access Port (Пользовательский порт)

Назначение:

Подключение конечных устройств - компьютеров, принтеров, IP-камер, серверов

Характеристики:

- Передает трафик только одного VLAN

- Не добавляет теги VLAN к кадрам

- "Верит" устройству на другом конце

Настройка:

bash

interface fastethernet 0/1

switchport mode access

switchport access vlan 72

Trunk Port (Магистральный порт)

Назначение:

Соединение между сетевыми устройствами - коммутаторами, маршрутизаторами

Характеристики:

- Передает трафик нескольких VLAN одновременно

- Добавляет теги VLAN (802.1Q) к кадрам

- "Не верит" устройству на другом конце

Настройка:

bash

interface gigabitethernet 0/1

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,72

- Пользовательский режим (Router>)

- Привилегированный режим (Router#)

- Режим глобальной конфигурации (Router(config)#)

Router> enable

Router# configure terminal (или Router# conf t)

Router(config)#

Что можно делать в режиме глобальной конфигурации:

Создание VLAN (виртуальных локальных сетей):

Router(config)# vlan 10

Router(config-vlan)# name Sales

Настройка протоколов маршрутизации (OSPF, EIGRP):

Router(config)# router ospf 1

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0

Настройка hostname устройства:

Router(config)# hostname MyOffice-Switch

MyOffice-Switch(config)#

Hostname устройства — это его сетевое имя, которое используется для идентификации в сети.

Настройка имени хоста

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch13 Switch13(config)#

Настройка парольной защиты

Switch13(config)#enable password iu3 Switch13(config)#exit Switch13# Switch13#show run Building configuration... Current configuration : 1104 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Switch13 ! enable password iu3 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! interface FastEthernet0/1 --More--

Errors (надо нажать Ctrl+Shift+6)

Switch13# ^ Translating "^"...domain server (255.255.255.255) % Name lookup aborted Switch13#

Настройка зашифрованного пароля доступа к привилегированному режиму

Switch13>enable Password: Switch13#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch13(config)#enable secret cisco Switch13(config)#do show run Building configuration... Current configuration : 1151 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Switch13 ! enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0 enable password iu3 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! --More--

После добавления пароля secret простой пароль уже не работает.

VTY (Virtual Teletype) — это виртуальные линии для удаленного доступа к устройству по протоколам Telnet и SSH.

Простыми словами: это "виртуальные дверцы" через которые можно подключиться к маршрутизатору или коммутатору удаленно по сети, а не через физическую консоль.

Что означают цифры 0 15?

- 0 15 означает, что мы настраиваем 16 виртуальных линий (от 0 до 15 включительно)

- Это позволяет одновременно подключиться к устройству 16 разным пользователям по Telnet/SSH

Настройка пароля линий VTY

Switch13>enable Password: Switch13#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch13(config)#line vty 0 15 Switch13(config-line)#password vty Switch13(config-line)#login Switch13(config-line)#do show run ¦ section line line con 0 line vty 0 4 password vty login line vty 5 15 password vty login

Служба шифрования пароля

Switch13(config-line)#do show run ¦ section line line con 0 line vty 0 4 password vty login line vty 5 15 password vty login Switch13#show run Building configuration... Current configuration : 1179 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec no service password-encryption ! hostname Switch13 ! enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0 enable password iu3 ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id Switch13#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch13(config)#service password-encryption Switch13(config)# Switch13# %SYS-5-CONFIG_I: Configured from console by console Switch13#show run Building configuration... Current configuration : 1197 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname Switch13 ! enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0 enable password 7 0828591D ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! Switch13#show run ¦ section line line con 0 line vty 0 4 password 7 08375857 login line vty 5 15 password 7 08375857 login

Настройка ip-адреса коммутатора

Switch13#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch13(config)#interface vlan 1 Switch13(config-if)#ip address 192.168.13.13 255.255.255.0 Switch13(config-if)#do show run Building configuration... Current configuration : 1222 bytes ! version 15.0 no service timestamps log datetime msec no service timestamps debug datetime msec service password-encryption ! hostname Switch13 ! enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0 enable password 7 0828591D ! ! ! ! ! ! spanning-tree mode pvst spanning-tree extend system-id ! interface FastEthernet0/1 … interface GigabitEthernet0/2 ! interface Vlan1 ip address 192.168.13.13 255.255.255.0 shutdown ! line con 0 ! line vty 0 4 password 7 08375857 login line vty 5 15 password 7 08375857 Switch13(config-if)#no shutdown Switch13(config)#do show interface vlan 1 Vlan1 is up, line protocol is up Hardware is CPU Interface, address is 0002.16ca.088e (bia 0002.16ca.088e) Internet address is 192.168.13.13/24 MTU 1500 bytes, BW 100000 Kbit, DLY 1000000 usec, reliability 255/255, txload 1/255, rxload 1/255 Encapsulation ARPA, loopback not set ARP type: ARPA, ARP Timeout 04:00:00 Last input 21:40:21, output never, output hang never Last clearing of "show interface" counters never Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0 Queueing strategy: fifo Output queue: 0/40 (size/max) 5 minute input rate 0 bits/sec, 0 packets/sec 5 minute output rate 0 bits/sec, 0 packets/sec 1682 packets input, 530955 bytes, 0 no buffer Received 0 broadcasts (0 IP multicast) 0 runts, 0 giants, 0 throttles 0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored 563859 packets output, 0 bytes, 0 underruns 0 output errors, 23 interface resets 0 output buffer failures, 0 output buffers swapped out Switch13#show interfaces fastethernet 0/2 FastEthernet0/2 is up, line protocol is up (connected) Hardware is Lance, address is 0050.0fc2.ce02 (bia 0050.0fc2.ce02) BW 100000 Kbit, DLY 1000 usec, reliability 255/255, txload 1/255, rxload 1/255 Encapsulation ARPA, loopback not set Keepalive set (10 sec) Full-duplex, 100Mb/s input flow-control is off, output flow-control is off ARP type: ARPA, ARP Timeout 04:00:00 Last input 00:00:08, output 00:00:05, output hang never Last clearing of "show interface" counters never Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0 Queueing strategy: fifo Output queue :0/40 (size/max) 5 minute input rate 0 bits/sec, 0 packets/sec 5 minute output rate 0 bits/sec, 0 packets/sec 956 packets input, 193351 bytes, 0 no buffer Received 956 broadcasts, 0 runts, 0 giants, 0 throttles 0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 0 abort 0 watchdog, 0 multicast, 0 pause input 0 input packets with dribble condition detected 2357 packets output, 263570 bytes, 0 underruns Desctop -> command promt Cisco Packet Tracer PC Command Line 1.0 C:\>ping 192.168.13.13 Pinging 192.168.13.13 with 32 bytes of data: Request timed out. Request timed out. Request timed out. Request timed out. Ping statistics for 192.168.13.13: Packets: Sent = 4, Received = 0, Lost = 4 (100% loss), C:\>ipconfig FastEthernet0 Connection:(default port) Connection-specific DNS Suffix..: Link-local IPv6 Address.........: FE80::2D0:D3FF:FE18:E04A IPv6 Address....................: :: IPv4 Address....................: 0.0.0.0 Subnet Mask.....................: 0.0.0.0 Default Gateway.................: :: 0.0.0.0 Bluetooth Connection: Connection-specific DNS Suffix..: Link-local IPv6 Address.........: :: IPv6 Address....................: :: IPv4 Address....................: 0.0.0.0 Subnet Mask.....................: 0.0.0.0 Default Gateway.................: :: 0.0.0.0 C:\>ipconfig FastEthernet0 Connection:(default port) Connection-specific DNS Suffix..: Link-local IPv6 Address.........: FE80::2D0:D3FF:FE18:E04A IPv6 Address....................: :: IPv4 Address....................: 192.168.13.10 Subnet Mask.....................: 255.255.255.0 Default Gateway.................: :: 0.0.0.0 Bluetooth Connection: Connection-specific DNS Suffix..: Link-local IPv6 Address.........: :: IPv6 Address....................: :: IPv4 Address....................: 0.0.0.0 Subnet Mask.....................: 0.0.0.0 Default Gateway.................: :: 0.0.0.0 C:\>ping 192.168.13.13 Pinging 192.168.13.13 with 32 bytes of data: Request timed out. Reply from 192.168.13.13: bytes=32 time<1ms TTL=255 Reply from 192.168.13.13: bytes=32 time<1ms TTL=255 Reply from 192.168.13.13: bytes=32 time<1ms TTL=255 Ping statistics for 192.168.13.13: Packets: Sent = 4, Received = 3, Lost = 1 (25% loss), Approximate round trip times in milli-seconds: Minimum = 0ms, Maximum = 0ms, Average = 0ms C:\>telnet 192.168.13.13 Trying 192.168.13.13 ...Open User Access Verification Password: vty Switch13>

Настройка шлюза по умолчанию

НА роутере --- System Configuration Dialog --- Would you like to enter the initial configuration dialog? [yes/no]: % Please answer 'yes' or 'no'. Would you like to enter the initial configuration dialog? [yes/no]: yes At any point you may enter a question mark '?' for help. Use ctrl-c to abort configuration dialog at any prompt. Default settings are in square brackets '[]'. Basic management setup configures only enough connectivity for management of the system, extended setup will ask you to configure each interface on the system Would you like to enter basic management setup? [yes/no]: y Configuring global parameters: Enter host name [Router]: Router13 The enable secret is a password used to protect access to privileged EXEC and configuration modes. This password, after entered, becomes encrypted in the configuration. Enter enable secret: cisco The enable password is used when you do not specify an enable secret password, with some older software versions, and some boot images. Enter enable password: cisco % Please choose a password that is different from the enable secret Enter enable password: 1234 The virtual terminal password is used to protect access to the router over a network interface. Enter virtual terminal password: vty Configure SNMP Network Management? [no]: Current interface summary Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0/0 unassigned YES manual administratively down down GigabitEthernet0/0/1 unassigned YES manual administratively down down GigabitEthernet0/0/2 unassigned YES manual administratively down down Vlan1 unassigned YES manual administratively down down Enter interface name used to connect to the management network from the above interface summary: GigabitEthernet0/0/0 Configuring interface GigabitEthernet0/0/0: Configure IP on this interface? [yes]: y IP address for this interface: 192.168.13.1 Subnet mask for this interface [255.255.255.0] : The following configuration command script was created: ! hostname Router13 enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0 enable password 1234 line vty 0 4 password vty ! interface Vlan1 shutdown no ip address ! interface GigabitEthernet0/0/0 no shutdown ip address 192.168.13.1 255.255.255.0 ! interface GigabitEthernet0/0/1 shutdown no ip address ! interface GigabitEthernet0/0/2 shutdown no ip address ! end [0] Go to the IOS command prompt without saving this config. [1] Return back to the setup without saving this config. [2] Save this configuration to nvram and exit. Enter your selection [2]: Building configuration... [OK] Use the enabled mode 'configure' command to modify this configuration. Press RETURN to get started! %LINK-5-CHANGED: Interface Vlan1, changed state to up %LINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up %LINK-5-CHANGED: Interface GigabitEthernet0/0/1, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/1, changed state to up %LINK-5-CHANGED: Interface GigabitEthernet0/0/2, changed state to up %LINK-5-CHANGED: Interface Vlan1, changed state to administratively down %LINK-5-CHANGED: Interface GigabitEthernet0/0/1, changed state to administratively down %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/1, changed state to down %LINK-5-CHANGED: Interface GigabitEthernet0/0/2, changed state to administratively down %SYS-5-CONFIG_I: Configured from console by console Router13>enable Password: cisco Router13#show interfaces gigabitethernet0/0/0 GigabitEthernet0/0/0 is up, line protocol is up (connected) Hardware is ISR4331-3x1GE, address is 00e0.f79e.7701 (bia 00e0.f79e.7701) Internet address is 192.168.13.1/24 MTU 1500 bytes, BW 1000000 Kbit, DLY 10 usec, reliability 255/255, txload 1/255, rxload 1/255 Encapsulation ARPA, loopback not set Keepalive not supported Full Duplex, 1000Mbps, link type is auto, media type is Auto Select output flow-control is on, input flow-control is on ARP type: ARPA, ARP Timeout 04:00:00, Last input 00:00:08, output 00:00:05, output hang never Last clearing of "show interface" counters never Input queue: 0/375/0 (size/max/drops); Total output drops: 0 Queueing strategy: fifo Output queue :0/40 (size/max) 5 minute input rate 0 bits/sec, 0 packets/sec 5 minute output rate 0 bits/sec, 0 packets/sec 0 packets input, 0 bytes, 0 no buffer Received 0 broadcasts (0 IP multicasts) 0 runts, 0 giants, 0 throttles 0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored 0 watchdog, 1017 multicast, 0 pause input Router13#conf t Enter configuration commands, one per line. End with CNTL/Z. Router13(config)#interface gigabitethernet0/0/1 Router13(config-if)#no shutdown Router13(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/0/1, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/1, changed state to up Теперь соединение зеленое Router13#show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0/0 192.168.13.1 YES manual up up GigabitEthernet0/0/1 unassigned YES unset up up GigabitEthernet0/0/2 unassigned YES unset administratively down down Vlan1 unassigned YES unset administratively down down Router13#conf t Enter configuration commands, one per line. End with CNTL/Z. Router13(config)#interface gigabitethernet0/0/1 Router13(config-if)#ip address 192.168.14.1 255.255.255.0 Router13(config-if)#no shutdown Router13(config)#ip routing Router13(config)#do show ip interface brief Interface IP-Address OK? Method Status Protocol GigabitEthernet0/0/0 192.168.13.1 YES manual up up GigabitEthernet0/0/1 192.168.14.1 YES manual up up GigabitEthernet0/0/2 unassigned YES unset administratively down down Vlan1 unassigned YES unset administratively down down На PC3 выполните: bash C:\> ping 192.168.13.10 Ожидаемый результат: ❌ Узел НЕ ДОСТУПЕН (будет Request timed out) Почему узел не доступен? Причина: - PC3 находится в сети 192.168.14.0/24 - PC1 находится в сети 192.168.13.0/24 - Коммутатор Switch13 не знает маршрут к сети 192.168.14.0/24 Решение: Нужно настроить шлюз по умолчанию на PC1 и Switch13: На PC1: - Desktop → IP Configuration - Default Gateway: 192.168.13.1 На Switch13: bash Switch13# configure terminal Switch13(config)# ip default-gateway 192.168.13.1 Switch13(config)# exit Switch13>enable Password: Switch13#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch13(config)#ip default-gateway 192.168.13.1 Настройка IP-адреса на PC3: Шаг 1: Настройте статический IP на PC3 - Закройте Command Prompt на PC3 - Перейдите: Desktop → IP Configuration - Убедитесь, что выбран FastEthernet0 (в выпадающем списке вверху) - Выберите "Static" и введите: - IP Address: 192.168.14.10 - Subnet Mask: 255.255.255.0 - Default Gateway: 192.168.14.1 C:\>ipconfig FastEthernet0 Connection:(default port) Connection-specific DNS Suffix..: Link-local IPv6 Address.........: FE80::240:BFF:FECA:A691 IPv6 Address....................: :: IPv4 Address....................: 192.168.14.10 Subnet Mask.....................: 255.255.255.0 Default Gateway.................: :: 192.168.14.1 Bluetooth Connection: Connection-specific DNS Suffix..: Link-local IPv6 Address.........: :: IPv6 Address....................: :: IPv4 Address....................: 0.0.0.0 Subnet Mask.....................: 0.0.0.0 Default Gateway.................: :: 0.0.0.0 Проверка связи с маршрутизатором C:\>ping 192.168.14.1 Pinging 192.168.14.1 with 32 bytes of data: Reply from 192.168.14.1: bytes=32 time=11ms TTL=255 Reply from 192.168.14.1: bytes=32 time<1ms TTL=255 Reply from 192.168.14.1: bytes=32 time<1ms TTL=255 Reply from 192.168.14.1: bytes=32 time=15ms TTL=255 Ping statistics for 192.168.14.1: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 0ms, Maximum = 15ms, Average = 6ms Первый пакет потерялся PC3 не знает MAC-адрес PC1 Первый ping - это ARP-запрос для поиска MAC-адреса Последующие пакеты идут нормально, так как ARP-таблица заполнена C:\>ping 192.168.13.10 Pinging 192.168.13.10 with 32 bytes of data: Request timed out. Reply from 192.168.13.10: bytes=32 time<1ms TTL=127 Reply from 192.168.13.10: bytes=32 time<1ms TTL=127 Reply from 192.168.13.10: bytes=32 time<1ms TTL=127 Ping statistics for 192.168.13.10: Packets: Sent = 4, Received = 3, Lost = 1 (25% loss), Approximate round trip times in milli-seconds: Minimum = 0ms, Maximum = 0ms, Average = 0ms C:\>ping 192.168.13.10 Pinging 192.168.13.10 with 32 bytes of data: Reply from 192.168.13.10: bytes=32 time<1ms TTL=127 Reply from 192.168.13.10: bytes=32 time<1ms TTL=127 Reply from 192.168.13.10: bytes=32 time<1ms TTL=127 Reply from 192.168.13.10: bytes=32 time=25ms TTL=127 Ping statistics for 192.168.13.10: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 0ms, Maximum = 25ms, Average = 6ms

Сохранение конфигурации

Switch13#copy running-config startup-config Destination filename [startup-config]? Building configuration... [OK] Switch13#

Настройка баннера входа

Switch13#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch13(config)#banner motd % Enter TEXT message. End with the character '%'. WARNING - WARNING - WARNING - WARNING - WARNING - WARNING - WARNING% Switch13#logout Switch13 con0 is now available Press RETURN to get started. WARNING - WARNING - WARNING - WARNING - WARNING - WARNING - WARNING Switch13>enable Password: Switch13#

Первая проба

A summary of U.S. laws governing Cisco cryptographic products may be found at:

http://www.cisco.com/wwl/export/crypto/tool/stqrg.html

If you require further assistance please contact us by sending email to

export@cisco.com.

cisco WS-C2960-24TT-L (PowerPC405) processor (revision B0) with 65536K bytes of memory.

Processor board ID FOC1010X104

Last reset from power-on

1 Virtual Ethernet interface

24 FastEthernet interfaces

2 Gigabit Ethernet interfaces

The password-recovery mechanism is enabled.

64K bytes of flash-simulated non-volatile configuration memory.

Base ethernet MAC Address : 00:D0:97:C5:98:9B

Motherboard assembly number : 73-10390-03

Power supply part number : 341-0097-02

Motherboard serial number : FOC10093R12

Power supply serial number : AZS1007032H

Model revision number : B0

Motherboard revision number : B0

Model number : WS-C2960-24TT-L

System serial number : FOC1010X104

Top Assembly Part Number : 800-27221-02

Top Assembly Revision Number : A0

Version ID : V02

CLEI Code Number : COM3L00BRA

Hardware Board Revision Number : 0x01

Switch Ports Model SW Version SW Image

------ ----- ----- ---------- ----------

* 1 26 WS-C2960-24TT-L 15.0(2)SE4 C2960-LANBASEK9-M

Cisco IOS Software, C2960 Software (C2960-LANBASEK9-M), Version 15.0(2)SE4, RELEASE SOFTWARE (fc1)

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2013 by Cisco Systems, Inc.

Compiled Wed 26-Jun-13 02:49 by mnguyen

Press RETURN to get started!

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#

Switch con0 is now available

Press RETURN to get started.

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname SwitchX

SwitchX(config)#enable password 1234

SwitchX(config)#show running-config

^

% Invalid input detected at '^' marker.

SwitchX(config)#show running-config

^

% Invalid input detected at '^' marker.

SwitchX(config)#show running-config

^

% Invalid input detected at '^' marker.

SwitchX(config)# show running-config

^

% Invalid input detected at '^' marker.

SwitchX(config)#

SwitchX(config)#

SwitchX#

%SYS-5-CONFIG_I: Configured from console by console

SwitchX#show running-config

Building configuration...

Current configuration : 1104 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname SwitchX

!

enable password 1234

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

SwitchX#enable

SwitchX#disable

SwitchX>

SwitchX>enable

Password:

SwitchX#enable secret 12345

^

% Invalid input detected at '^' marker.

SwitchX#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

SwitchX(config)#enable secret 12345

SwitchX(config)#

SwitchX#

%SYS-5-CONFIG_I: Configured from console by console

SwitchX#show running-config

Building configuration...

Current configuration : 1151 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname SwitchX

!

enable secret 5 $1$mERr$/Q/mbs3O9oHsKR7rNG4e81

enable password 1234

!

!

!

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

SwitchX#disable

SwitchX>enable secret 12345

^

% Invalid input detected at '^' marker.

SwitchX>enable

Password:

SwitchX#line vty 0 15

^

% Invalid input detected at '^' marker.

SwitchX#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

SwitchX(config)#line vty 0 15

SwitchX(config-line)#password 1234

SwitchX(config-line)#login

SwitchX(config-line)#running-condig

^

% Invalid input detected at '^' marker.

SwitchX(config-line)#

SwitchX#

%SYS-5-CONFIG_I: Configured from console by console

SwitchX#running config

^

% Invalid input detected at '^' marker.

SwitchX#running-config

Translating "running-config"...domain server (255.255.255.255)