Сохранение:

copy running-config startup-config

Топология cisco Packet Tracer:

Switch0 -> Switch1

Switch0 -> Switch2

Switch0 -> PC1

Switch1 -> PC0

Switch2 -> PC2

Switch2 -> Switch3

Switch3 -> PC3

Router0 -> Switch0

Switch0 – VTP Server

Switch1 – Client

Switch2 – Transparent

Switch3 – Client

Vlan72 – PC0, PC3

Vlan27 – PC1, PC2

Router0 -> Switch4

Switch4 -> Switch5

Switch4 -> Switch6

Switch4 -> PC7

Switch5 -> PC4

Switch6 -> PC6

Switch6 -> PC5

Vlan72 – PC7, PC6

Vlan27 – PC4, PC5

STP root vlan 72 - Switch5

STP root vlan 27 – Switch6

Errors (надо нажать Ctrl+Shift+6)

Switch13# ^ Translating "^"...domain server (255.255.255.255) % Name lookup aborted Switch13#

Switch0

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch0 ! Настраиваем транки к другим свитчам Switch0(config)#interface range fastethernet 0/1-3 Switch0(config-if-range)#switchport mode trunk Switch0(config-if-range)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up Switch0(config-if-range)#no shutdown ! Настраиваем порт к роутеру (будет настроен позже) Switch0(config-if-range)#interface fastethernet 0/4 Switch0(config-if)#no shutdown Switch0(config)#do show ip interface brief Interface IP-Address OK? Method Status Protocol FastEthernet0/1 unassigned YES manual up up FastEthernet0/2 unassigned YES manual up up FastEthernet0/3 unassigned YES manual up up FastEthernet0/4 unassigned YES manual down down … Vlan1 unassigned YES manual administratively down down VTP Switch0(config)#vtp domain LAB Changing VTP domain name from NULL to LAB Switch0(config)#vtp password 1234 Setting device VLAN database password to 1234 Switch0(config)#vtp mode server Device mode already VTP SERVER. Switch0(config)#vtp version 2 Создаем VLAN (после назначения ролей VTP) Switch0(config)#vlan 72 Switch0(config-vlan)#name VLAN72 Switch0(config-vlan)#exit Switch0(config)#vlan 27 Switch0(config-vlan)#name VLAN27 Switch0(config-vlan)#exit Switch0(config)#vtp version 2 Switch0(config)#do show vtp status VTP Version capable : 1 to 2 VTP version running : 2 VTP Domain Name : LAB VTP Pruning Mode : Disabled VTP Traps Generation : Disabled Device ID : 0003.E405.A500 Configuration last modified by 0.0.0.0 at 3-1-93 00:48:58 Local updater ID is 0.0.0.0 (no valid interface found) Feature VLAN : -------------- VTP Operating Mode : Server Maximum VLANs supported locally : 255 Number of existing VLANs : 7 Configuration Revision : 6 MD5 digest : 0x66 0x14 0x36 0x61 0x17 0x18 0xAC 0x6A 0x00 0x58 0xCC 0xDF 0xAD 0x0E 0x63 0x02 Switch0(config)#do show vlan brief VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa0/1, Fa0/4, Fa0/5, Fa0/6 Fa0/7, Fa0/8, Fa0/9, Fa0/10 Fa0/11, Fa0/12, Fa0/13, Fa0/14 Fa0/15, Fa0/16, Fa0/17, Fa0/18 Fa0/19, Fa0/20, Fa0/21, Fa0/22 Fa0/23, Fa0/24, Gig0/1, Gig0/2 27 VLAN27 active 72 VLAN72 active 1002 fddi-default active 1003 token-ring-default active 1004 fddinet-default active 1005 trnet-default active Switch0(config)#interface fastethernet 0/1 Switch0(config-if)#switchport mode access Switch0(config-if)#switchport access vlan 27 Порт к роутеру Switch0(config)#interface fastethernet 0/4 Switch0(config-if)#switchport mode trunk Switch0(config-if)#no shutdown

Switch1

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch1 Switch1(config)#interface fastethernet 0/2 Switch1(config-if)#switchport mode trunk Switch1(config-if)#no shutdown Switch1(config-if)#exit Switch1(config)#interface fastethernet 0/1 Switch1(config-if)#switchport mode access Switch1(config-if)#no shutdown Switch1(config-if)#exit VTP Switch1(config)#vtp domain LAB Domain name already set to LAB. Switch1(config)#vtp password 1234 Setting device VLAN database password to 1234 Switch1(config)#vtp mode client Setting device to VTP CLIENT mode. После VLAN Switch1(config)#do show vtp status VTP Version capable : 1 to 2 VTP version running : 2 VTP Domain Name : LAB VTP Pruning Mode : Disabled VTP Traps Generation : Disabled Device ID : 00D0.97D0.9C00 Configuration last modified by 0.0.0.0 at 3-1-93 00:48:58 Feature VLAN : -------------- VTP Operating Mode : Client Maximum VLANs supported locally : 255 Number of existing VLANs : 7 Configuration Revision : 6 MD5 digest : 0x66 0x14 0x36 0x61 0x17 0x18 0xAC 0x6A 0x00 0x58 0xCC 0xDF 0xAD 0x0E 0x63 0x02 Switch1(config)#do show vlan brief VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa0/1, Fa0/3, Fa0/4, Fa0/5 Fa0/6, Fa0/7, Fa0/8, Fa0/9 Fa0/10, Fa0/11, Fa0/12, Fa0/13 Fa0/14, Fa0/15, Fa0/16, Fa0/17 Fa0/18, Fa0/19, Fa0/20, Fa0/21 Fa0/22, Fa0/23, Fa0/24, Gig0/1 Gig0/2 27 VLAN27 active 72 VLAN72 active 1002 fddi-default active 1003 token-ring-default active 1004 fddinet-default active 1005 trnet-default active На Switch1 назначаем порт для PC0 в VLAN 72: Switch1(config)#interface fastethernet 0/1 Switch1(config-if)#switchport mode access Switch1(config-if)#switchport access vlan 72

Switch2

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch2 Switch2(config)#interface range fastethernet 0/2-3 Switch2(config-if-range)#switchport mode trunk Switch2(config-if-range)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/2, changed state to up Switch2(config-if-range)#no shutdown Switch2(config-if-range)#exit Switch2(config)#interface fastethernet 0/1 Switch2(config-if)#switchport mode access Switch2(config-if)#no shutdown Switch2(config-if)#exit VTP Switch2(config)#vtp domain LAB Domain name already set to LAB. Switch2(config)#vtp password 1234 Setting device VLAN database password to 1234 Switch2(config)#vtp mode transparent Setting device to VTP TRANSPARENT mode. Switch2(config)#vtp version 2 Switch2(config)#do show vtp status VTP Version capable : 1 to 2 VTP version running : 2 VTP Domain Name : LAB VTP Pruning Mode : Disabled VTP Traps Generation : Disabled Device ID : 000B.BE68.7000 Configuration last modified by 0.0.0.0 at 3-1-93 00:54:03 Feature VLAN : -------------- VTP Operating Mode : Transparent Maximum VLANs supported locally : 255 Number of existing VLANs : 5 Configuration Revision : 0 MD5 digest : 0x24 0xD4 0x08 0x97 0x6A 0x26 0x49 0x5F 0x3A 0xD1 0x35 0x1B 0xA0 0x15 0xA4 0x55 Switch2(config)#do show vlan brief VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa0/1, Fa0/4, Fa0/5, Fa0/6 Fa0/7, Fa0/8, Fa0/9, Fa0/10 Fa0/11, Fa0/12, Fa0/13, Fa0/14 Fa0/15, Fa0/16, Fa0/17, Fa0/18 Fa0/19, Fa0/20, Fa0/21, Fa0/22 Fa0/23, Fa0/24, Gig0/1, Gig0/2 1002 fddi-default active 1003 token-ring-default active 1004 fddinet-default active 1005 trnet-default active ТУТ нет настроеных VALN сервером т к Transparent На Switch2 (Transparent) - создаем VLAN локально и назначаем порты: Switch2(config)#vlan 72 Switch2(config-vlan)#name VLAN72 Switch2(config-vlan)#exit Switch2(config)#vlan 27 Switch2(config-vlan)#name VLAN27 Switch2(config-vlan)#exit Switch2(config)#interface fastethernet 0/1 Switch2(config-if)#switchport mode access Switch2(config-if)#switchport access vlan 27

Switch3

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch3 Switch3(config)#interface fastethernet 0/2 Switch3(config-if)#switchport mode trunk Switch3(config-if)#no shutdown Switch3(config-if)#exit Switch3(config)#interface fastethernet 0/1 Switch3(config-if)#switchport mode access Switch3(config-if)#no shutdown Switch3(config-if)#exit VTP Switch3(config)#vtp domain LAB Domain name already set to LAB. Switch3(config)#vtp password 1234 Setting device VLAN database password to 1234 Switch3(config)#vtp mode client Setting device to VTP CLIENT mode. Switch3(config)#interface fastethernet 0/1 Switch3(config-if)#switchport mode access Switch3(config-if)#switchport access vlan 72

Router0

Router>enable Router#conf t Enter configuration commands, one per line. End with CNTL/Z. Router(config)#hostname Router0 Router0(config)#interface gigabitethernet 0/0 Router0(config-if)#no shutdown Router0(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up Router0(config-if)#interface gigabitethernet 0/0.72 Router0(config-subif)# %LINK-5-CHANGED: Interface GigabitEthernet0/0.72, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0.72, changed state to up Router0(config-subif)#encapsulation dot1Q 72 Router0(config-subif)#ip address 192.168.1.1 255.255.255.0 Router0(config-subif)#no shutdown Router0(config-subif)#exit Router0(config)#interface gigabitethernet 0/0.27 Router0(config-subif)# %LINK-5-CHANGED: Interface GigabitEthernet0/0.27, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0.27, changed state to up Router0(config-subif)#encapsulation dot1Q 27 Router0(config-subif)#ip address 192.168.2.1 255.255.255.0 Router0(config-subif)#no shutdown

Switch4

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch4 Switch4(config)#vtp domain LAB Changing VTP domain name from NULL to LAB Switch4(config)#vtp password 1234 Setting device VLAN database password to 1234 Switch4(config)#vtp mode server Device mode already VTP SERVER. Switch4(config)#vtp version 2 Switch4(config)#vlan 72 Switch4(config-vlan)#name VLAN72 Switch4(config-vlan)#exit Switch4(config)#vlan 27 Switch4(config-vlan)#name VLAN27 Switch4(config-vlan)#exit Switch4(config)#interface range fastethernet 0/2-4 Switch4(config-if-range)#switchport mode trunk Switch4(config-if-range)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/4, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/4, changed state to up Switch4(config-if-range)#no shutdown Switch4(config-if-range)#exit Switch4(config)#interface fastethernet 0/1 Switch4(config-if)#switchport mode access Switch4(config-if)#switchport access vlan 72 Switch4(config-if)#no shutdown Switch4(config-if)

Switch5

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch5 Switch5(config)#vtp domain LAB Changing VTP domain name from NULL to LAB Switch5(config)#vtp password 1234 Setting device VLAN database password to 1234 Switch5(config)#vtp mode client Setting device to VTP CLIENT mode. Switch5(config)#do show vlan brief VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa0/1, Fa0/3, Fa0/4, Fa0/5 Fa0/6, Fa0/7, Fa0/8, Fa0/9 Fa0/10, Fa0/11, Fa0/12, Fa0/13 Fa0/14, Fa0/15, Fa0/16, Fa0/17 Fa0/18, Fa0/19, Fa0/20, Fa0/21 Fa0/22, Fa0/23, Fa0/24, Gig0/1 Gig0/2 27 VLAN27 active 72 VLAN72 active 1002 fddi-default active 1003 token-ring-default active 1004 fddinet-default active 1005 trnet-default active Switch5(config)#interface range fastethernet 0/2-3 Switch5(config-if-range)#switchport mode trunk Switch5(config-if-range)# %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to down %LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/3, changed state to up Switch5(config-if-range)#no shutdown Switch5(config-if-range)#exit Switch5(config)#interface fastethernet 0/1 Switch5(config-if)#switchport mode access Switch5(config-if)#switchport access vlan 27 Switch5(config-if)#no shutdown Switch5(config-if)#exit STP Switch5(config)#spanning-tree mode rapid-pvst Switch5(config)#spanning-tree vlan 72 root primary Switch5(config)#spanning-tree vlan 27 root secondary Switch5(config)#interface fastethernet 0/1 Switch5(config-if)#spanning-tree portfast

Switch6

Switch>enable Switch#conf t Enter configuration commands, one per line. End with CNTL/Z. Switch(config)#hostname Switch6 Switch6(config)#vtp domain LAB Domain name already set to LAB. Switch6(config)#vtp password 1234 Setting device VLAN database password to 1234 Switch6(config)#vtp mode client Setting device to VTP CLIENT mode. Switch6(config)#do show vlan brief VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa0/1, Fa0/2, Fa0/5, Fa0/6 Fa0/7, Fa0/8, Fa0/9, Fa0/10 Fa0/11, Fa0/12, Fa0/13, Fa0/14 Fa0/15, Fa0/16, Fa0/17, Fa0/18 Fa0/19, Fa0/20, Fa0/21, Fa0/22 Fa0/23, Fa0/24, Gig0/1, Gig0/2 27 VLAN27 active 72 VLAN72 active 1002 fddi-default active 1003 token-ring-default active 1004 fddinet-default active 1005 trnet-default active Switch6(config)#interface range fastethernet 0/3-4 Switch6(config-if-range)#switchport mode trunk Switch6(config-if-range)#no shutdown Switch6(config-if-range)#exit Switch6(config)#interface range fastethernet 0/1-2 Switch6(config-if-range)#switchport mode access Switch6(config-if-range)#exit Switch6(config)#interface fastethernet 0/1 Switch6(config-if)#switchport access vlan 72 Switch6(config-if)#no shutdown Switch6(config-if)#exit Switch6(config)#interface fastethernet 0/2 Switch6(config-if)#switchport access vlan 27 Switch6(config-if)#no shutdown Switch6(config-if)#exit Switch6(config)# STP Switch6(config)#spanning-tree mode rapid-pvst Switch6(config)#spanning-tree vlan 27 root primary Switch6(config)#spanning-tree vlan 72 root secondary Switch6(config)#interface fastethernet 0/1 Switch6(config-if)#spanning-tree portfast %Warning: portfast should only be enabled on ports connected to a single host. Connecting hubs, concentrators, switches, bridges, etc... to this interface when portfast is enabled, can cause temporary bridging loops. Use with CAUTION %Portfast has been configured on FastEthernet0/1 but will only have effect when the interface is in a non-trunking mode. Switch6(config-if)#exit Switch6(config)#interface fastethernet 0/2 Switch6(config-if)#spanning-tree portfast %Warning: portfast should only be enabled on ports connected to a single host. Connecting hubs, concentrators, switches, bridges, etc... to this interface when portfast is enabled, can cause temporary bridging loops. Use with CAUTION %Portfast has been configured on FastEthernet0/2 but will only have effect when the interface is in a non-trunking mode. Switch6(config-if)#exit

Router0

Router0(config)#interface gigabitethernet 0/1 Router0(config-if)#no shutdown Router0(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/1, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up Router0(config-if)#exit Router0(config)#interface gigabitethernet 0/1.72 Router0(config-subif)# %LINK-5-CHANGED: Interface GigabitEthernet0/1.72, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1.72, changed state to up Router0(config-subif)#encapsulation dot1Q 72 Router0(config-subif)#ip address 192.168.3.1 255.255.255.0 Router0(config-subif)#no shutdown Router0(config-subif)#exit Router0(config)#interface gigabitethernet 0/1.27 Router0(config-subif)# %LINK-5-CHANGED: Interface GigabitEthernet0/1.27, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1.27, changed state to up Router0(config-subif)#encapsulation dot1Q 27 Router0(config-subif)#ip address 192.168.4.1 255.255.255.0 Router0(config-subif)#no shutdown Router0(config-subif)#exit