Сохранение:

copy running-config startup-config

Топология cisco Packet Tracer:

Есть такая топология для лабы

(облака это отдельная IP-сеть)

AS 65001

172.23.0.0/16

Router0 - облако0

Router0 - облако1

Router1 - облако0

Router1 - облако2

Router1 - облако6

PC2 - облако6

Router2 - облако1

Router2 - облако3

Router3 - облако2

Router3 - облако3

Router3 - облако4

Router3 - облако5

PC0 - облако4

PC1 - облако5

Router1 - Router5 обьеденены по протоколы OSPF с редестребьюцией

Router0 – Router4

AS 65002

172.24.0.0/16

Router4 - облако7

Router4 - облако8

PC3 - облако7

PC4 - облако8

Router0 и Router4 - BGP

Errors (надо нажать Ctrl+Shift+6)

Switch13# ^ Translating "^"...domain server (255.255.255.255) % Name lookup aborted Switch13#

show ip route show ip interface brief show ip bgp

Router 0

Router>enable Router#conf t Enter configuration commands, one per line. End with CNTL/Z. Router(config)#hostname Router0 interface GigabitEthernet0/0 # к Cloud0 ip address 172.23.0.1 255.255.255.0 no shutdown ip ospf 1 area 0 interface GigabitEthernet0/1 # к Cloud1 ip address 172.23.1.1 255.255.255.0 ip ospf 1 area 0 no shutdown router ospf 1 network 172.23.0.0 0.0.255.255 area 0 redistribute bgp 65001 subnets interface GigabitEthernet0/2 ip address 172.20.0.1 255.255.0.0 no shutdown # BGP настройки: Router0(config)#router bgp 65001 Router0(config-router)#neighbor 172.20.0.2 remote-as 65002 Router0(config-router)#network 172.23.0.0 mask 255.255.0.0 Router0(config-router)#redistribute ospf 1 Router0(config-router)#neighbor 172.20.0.2 next-hop-self

Router 1

Router>enable Router#conf t Enter configuration commands, one per line. End with CNTL/Z. Router(config)#hostname Router1 interface GigabitEthernet0/0 # к Router0 (172.23.0.0/24) ip address 172.23.0.2 255.255.255.0 ip ospf 1 area 0 no shutdown interface GigabitEthernet0/1 # к Router3 (172.23.2.0/24) ip address 172.23.2.1 255.255.255.0 ip ospf 1 area 0 no shutdown interface GigabitEthernet0/2 # к PC2 ip address 172.23.6.1 255.255.255.0 no shutdown

Router 2

Router>enable Router#conf t Enter configuration commands, one per line. End with CNTL/Z. Router(config)#hostname Router2 Router2(config)#interface GigabitEthernet0/0 Router2(config-if)# ip address 172.23.1.1 255.255.255.0 Router2(config-if)#no shutdown Router2(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up %IP-4-DUPADDR: Duplicate address 172.23.1.1 on GigabitEthernet0/0, sourced by 000C.8557.C602 %IP-4-DUPADDR: Duplicate address 172.23.1.1 on GigabitEthernet0/0, sourced by 000C.8557.C602 Router2(config-if)#exit Router2(config)#interface GigabitEthernet0/1 Router2(config-if)# ip address 172.23.3.2 255.255.255.0 Router2(config-if)#no shutdown Router2(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/1, changed state to up

Router 3

Router>enable Router#conf t Enter configuration commands, one per line. End with CNTL/Z. Router(config)#hostname Router3 Router3(config)#int fa0/0 Router3(config-if)# ip address 172.23.2.3 255.255.255.0 Router3(config-if)#no shutdown Router3(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up Router3(config-if)#exit Router3(config)#int fa0/1 Router3(config-if)# ip address 172.23.3.3 255.255.255.0 Router3(config-if)#no shutdown Router3(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/1, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up Router3(config-if)#exit int fa1/0 ip address 172.23.4.1 255.255.255.0 no shutdown int fa1/1 ip address 172.23.5.1 255.255.255.0 no shutdown Router3(config)#router ospf 1 Router3(config-router)#network 172.23.0.0 0.0.255.255 area 0

Router4

interface GigabitEthernet0/0 ip address 172.20.0.2 255.255.0.0 no shutdown Router4(config)#router bgp 65002 Router4(config-router)#neighbor 172.20.0.1 remote-as 65001 Router4(config-router)#network 172.24.0.0 mask 255.255.0.0 Router4(config-router)#redistribute connected